MODULE 1: SLIDE TEXT CONTENT

Welcome to Your PEV Brain

This document contains ONLY the text that should appear on each slide. Use this as your reference when creating the PowerPoint.

SLIDE 1: Title Slide

MODULE 1

Welcome to Your PEV Brain

Introduction to CAN Bus Communication

Lectec PEV AI Curriculum

Day 1 of 14

SLIDE 2: The Hook

What If Your PEV Could Think?

Imagine riding down the street and your PEV automatically:

- Spots a pedestrian stepping into your path

- Warns you with a beep before you even see them

- Logs the near-miss for your safety review

By Day 14, YOU will build this system!

SLIDE 3: Real-World Technology

This Tech Powers Real Vehicles

Tesla Autopilot

Uses CAN bus to read 8 cameras, 12 ultrasonic sensors, and control steering

Waymo Robotaxi

Fully autonomous vehicles using the same CAN protocol you'll learn today

Your Lectec PEV

Same technology, your hands, real learning - starting today!

SLIDE 4: Learning Objectives

Today's Mission

What you'll accomplish in this module

- Explain what CAN bus is

Understand the 'nervous system' of your PEV

- Identify the three main hardware components

Raspberry Pi Zero 2W, VESC Controller, AI Camera

- Learn to use Jupyter Notebooks

Run code, stop code, and understand cells

- Read real data from your PEV using Python

Voltage, RPM, temperature - live and instant!

SLIDE 5: System Analogy

Your PEV Has a Nervous System

Human Body

- Brain = Processes information

- Nerves = Carry signals

- Eyes = See the world

- Muscles = Take action

Your PEV

- Raspberry Pi = The brain

- CAN Bus = The nerves

- AI Camera = The eyes

- VESC Motor = The muscles

SLIDE 6: Hardware Overview

Meet Your Hardware Team

THE BRAIN: Raspberry Pi Zero 2W

- 1GHz quad-core processor

- 512MB RAM

- Runs Python code

- WiFi built-in

THE MUSCLES: VESC Motor Controller

- Controls motor speed

- Reports RPM, voltage, temp

- CAN bus interface

- Real-time telemetry

THE EYES: Sony IMX500 AI Camera

- AI built INTO the chip!

- Detects 80 object types

- 30 FPS processing

- Low power consumption

SLIDE 7: CAN Bus Explained

CAN Bus: Controller Area Network

The universal language of vehicles

Key Facts

- Invented in 1986 by Bosch - originally for cars, now everywhere

- Used in every modern vehicle - cars, trucks, tractors, boats, planes

- Why it matters for YOU - industry-standard skill, real job applications

How It Works

- Two wires: CAN High + CAN Low

- All devices share one 'conversation'

- Messages have IDs for routing

SLIDE 8: System Architecture

How It All Connects

Your PEV's complete nervous system

Raspberry Pi Zero 2W <---> IMX500 AI Camera (via CSI Cable)

|

|

CAN BUS (2-Wire Network)

|

|

VESC Motor Controller

(Reports: Speed, RPM, Voltage, Temp)

SLIDE 9: Safety Rules

Safety First!

Essential guidelines for working with your PEV

- NEVER ride during coding - PEV must be on a stand or stationary

- Check battery before starting - Ensure above 20% for lab work

- Don't touch hot components - Motor and controller get warm

- Always call stop() when done - Clean shutdown protects hardware

- Ask before motor commands - Reading is safe, moving needs approval

- Report any strange behavior - Weird sounds/smells? Tell teacher!

Safety first = more fun with code later!

SLIDE 10: Jupyter Introduction

What is a Jupyter Notebook?

Your interactive coding environment

A Jupyter Notebook is like a smart document where you can:

- Write and run Python code

- See results immediately below your code

- Add notes and explanations

- Save your work and come back later

Interface Elements

- Cells = Individual boxes of code or text

- Run button = Play button that executes code

- Stop button = Square that stops running code

- Output area = Where results appear

Think of it like a recipe book where you can actually cook each step and see the result!

NOTEBOOK ACTIVITY Open your Jupyter Notebook now! Find the file called 'Module_01_PEV_Brain.ipynb'

SLIDE 11: Running Code

How to Run a Cell

Step 1: Click on a cell

The cell will get a blue or green border when selected

Step 2: Run using ONE of these methods:

- Click the Run button (play triangle) in the toolbar

- Press Shift + Enter on your keyboard

- Press Ctrl + Enter (runs but stays on same cell)

Step 3: Watch for the output

Results appear directly below the cell

The [ ] shows [*] while running, then [1], [2], etc. when done

NOTEBOOK ACTIVITY Go to Section A, Cell 1. Click on it and press Shift+Enter. You should see 'Welcome to Jupyter!' appear below.

SLIDE 12: Stopping Code

Stopping Code That Won't Stop

Sometimes code runs forever - here's how to stop it!

Why Code Might Run Forever

- Loops that never end (while True:)

- Waiting for something that never happens

- Accidentally created infinite loop

How to Stop

- Method 1: Click the Stop button (black square) in the toolbar

- Method 2: Press the 'I' key twice quickly (I, I)

- Method 3: Go to Kernel -> Interrupt (menu bar)

Signs Code is Still Running

- The cell shows [*] instead of a number

- You can't run other cells

- The circle in the top right is filled (busy)

NOTEBOOK ACTIVITY Go to Section A, Cell 2. This cell counts forever! Run it, watch it count for a few seconds, then STOP it using the Stop button.

SLIDE 13: What is an API?

What is an API?

Application Programming Interface

The Restaurant Analogy

Imagine you're at a restaurant:

- You (the customer) = Your Python code

- The Waiter = The API

- The Kitchen = The VESC motor controller

- The Menu = Available functions you can call

How It Works

- You look at the menu and tell the waiter what you want

- The waiter goes to the kitchen and places your order

- The kitchen prepares your food

- The waiter brings it back to you

You don't need to know HOW the kitchen makes the food.

You just need to know WHAT to ask for!

KEY INSIGHT: An API is a 'menu' of commands. You pick what you want, and the API handles all the complicated stuff behind the scenes.

SLIDE 14: The VESCStudentAPI

Meet Your API: VESCStudentAPI

Your menu of commands for talking to the VESC

Read Functions - Get info FROM the VESC

- get_input_voltage() - Battery voltage

- get_rpm() - Motor speed

- get_motor_current() - Electricity to motor

- get_fet_temperature() - Controller temp

- get_motor_temperature() - Motor temp

- ...and more!

Control Functions - Send commands TO the VESC

(Teacher approval required!)

- set_duty_cycle() - Control motor speed

- set_current() - Control motor torque

- set_brake_current() - Apply brakes

System Functions - Manage the connection

- start() - Connect to the VESC

- stop() - Disconnect safely

- is_connected() - Check if working

SLIDE 15: Connecting to VESC

Let's Connect to the VESC!

The Connection Code

from student_api import VESCStudentAPI vesc_api = VESCStudentAPI() vesc_api.start() vesc = vesc_api.get_controller(74)

Line-by-Line Explanation

- Line 1: Import the API (get the menu ready)

- Line 3: Create our API helper

- Line 4: Start the connection (walk into the restaurant)

- Line 6: Get access to controller #74 (our specific VESC)

Expected Output

Connected to VESC controller! Battery: XX.X V

If You See an Error

Check that the PEV is powered ON and the cable is connected

NOTEBOOK ACTIVITY Go to Section C in your notebook. Run the connection cell. Raise your hand when you see the battery voltage!

SLIDE 16: Reading Battery Voltage

Your First Data Read: Battery Voltage

The Code

voltage = vesc.get_input_voltage() print(f"Battery Voltage: {voltage} V")

What This Does

- Asks the VESC: 'What's the battery voltage right now?'

- VESC responds over CAN bus with the number

- We store it in a variable called 'voltage'

- We display it using print()

Understanding the Reading

- 36V+ = Fully charged (10S battery)

- 33-36V = Good charge remaining

- 30-33V = Getting low, consider charging

- Below 30V = Low battery warning!

NOTEBOOK ACTIVITY In Section D, Cell 1, write your own code to read voltage. Fill in the blank: voltage = vesc.________()

SLIDE 17: Reading RPM - Part 1

Reading Motor Speed (RPM)

RPM = Revolutions Per Minute

The Code

rpm = vesc.get_rpm() print(f"Motor RPM: {rpm}")

When motor is NOT spinning:

Output: Motor RPM: 0

This is normal! No movement = 0 RPM

When motor IS spinning:

Output: Motor RPM: 2847 (or some number)

Positive = Forward direction

Negative = Reverse direction

KEY POINT: The RPM reading is INSTANT - it shows what's happening RIGHT NOW.

NOTEBOOK ACTIVITY In Section D, Cell 2, run the RPM code with the motor NOT spinning. Write down: ______ RPM

SLIDE 18: Reading RPM - Part 2 (The Proof!)

Proving It's Real-Time!

Let's see the data change instantly

The Experiment

Step 1: Run the code with motor stopped

Expected result: 0 RPM

Write it down: ______ RPM

Step 2: Gently spin the wheel BY HAND

Keep it spinning slowly and safely

Step 3: While spinning, run the code AGAIN

Expected result: A number OTHER than 0!

Write it down: ______ RPM

The code reads what's happening RIGHT NOW!

This is called REAL-TIME DATA - instant feedback from your PEV.

NOTEBOOK ACTIVITY Run RPM cell, write number. GENTLY spin wheel by hand and run AGAIN while spinning. Write both numbers!

SLIDE 19: Fill-in-the-Blank: Temperature

Your Turn: Read the Temperature!

The Challenge

Fill in the blanks to read the controller (FET) temperature:

temp = vesc.get_____temperature() print(f"Controller Temperature: {____} C")

Hints

- Hint 1: The FET is the electronic part of the controller

- Hint 2: Look at the API list - what function reads FET temp?

- Hint 3: The second blank should be the variable name

Answer Check

Your output should look like: Controller Temperature: XX.X C

Normal reading: 20-50C when idle

NOTEBOOK ACTIVITY Go to Section E, Cell 1. Fill in the blanks and run. Write your temperature: ______ C

SLIDE 20: Matching Challenge

Match the Component to the Function!

What do you want to know?

- How fast is the motor spinning?

- How much battery do I have?

- Is the controller getting hot?

- How much electricity is the motor using?

- How hot is the motor itself?

Which function to use?

A. get_motor_temperature()

B. get_rpm()

C. get_input_voltage()

D. get_motor_current()

E. get_fet_temperature()

Answers: 1-B, 2-C, 3-E, 4-D, 5-A

NOTEBOOK ACTIVITY Complete the matching exercise in Section F of your notebook. Run the 'check answers' cell!

SLIDE 21: Summary

What You Learned Today

Achievement Checklist

- [DONE] CAN bus is the 'nervous system' of vehicles

- [DONE] Three components: Pi (brain), VESC (muscles), Camera (eyes)

- [DONE] How to use Jupyter Notebooks (run, stop, restart)

- [DONE] What an API is and why we use one

- [DONE] Connected to your PEV with Python

- [DONE] Read real voltage, RPM, and temperature data

- [DONE] Proved that the data is REAL-TIME

Key Terms

- CAN Bus - Controller Area Network, the two-wire communication system

- VESC - Motor speed controller that reports telemetry

- API - Application Programming Interface, your 'menu' of commands

- Telemetry - Data sent from the VESC (voltage, RPM, temp, etc.)

- Real-Time - Data that reflects what's happening RIGHT NOW

SLIDE 22: Coming Up Next

COMING UP IN MODULE 2

Building a Live Dashboard

Real-time data visualization with continuous updates

Preview

- Continuous monitoring loops

- Updating displays

- Data logging

Great job today!

You've taken your first step into vehicle communication

Today you asked single questions. Tomorrow, you'll have a continuous conversation!

JUPYTER NOTEBOOK CONTENT

The following sections detail the exact content for each cell in the companion Jupyter Notebook.

Section A: Jupyter Basics

Cell A.1 - Welcome (Markdown)

# Welcome to Jupyter Notebooks! This is your interactive coding environment. Let's learn how to use it!

Cell A.2 - First Code Cell

# Your first code cell! # Click on this cell, then press Shift+Enter to run it print("Welcome to Jupyter!") print("If you can see this, you ran the cell successfully!")

Cell A.3 - Counting Forever (Stop Practice)

# This code counts forever - you'll need to STOP it! import time counter = 1 while True: print(f"Counting: {counter}") counter = counter + 1 time.sleep(0.5)

Cell A.4 - Simple Math

# Python can do math! apples = 5 oranges = 3 total_fruit = apples + oranges print(f"I have {apples} apples") print(f"I have {oranges} oranges") print(f"Total fruit: {total_fruit}")

Cell A.5 - Variables Practice

# YOUR TURN: Change the values below and run again! my_name = "Student" # Change to your name! my_age = 15 # Change to your age! print(f"Hello, my name is {my_name}") print(f"I am {my_age} years old")

Section B: Functions Practice

Cell B.1 - Functions Introduction

# Functions are like recipes def say_hello(name): print(f"Hello, {name}!") print("Welcome to the PEV coding class!") say_hello("Everyone")

Cell B.2 - Function with Return

# Some functions give you a value back def add_numbers(a, b): result = a + b return result my_sum = add_numbers(10, 25) print(f"10 + 25 = {my_sum}")

Cell B.3 - Practice (Fill in Blank)

# Fill in the blank: multiply two numbers def multiply_numbers(a, b): result = a _____ b # What goes here? return result product = multiply_numbers(6, 7) print(f"6 x 7 = {product}") # Should print 42

Section C: Connecting to VESC

Cell C.1 - Header (Markdown)

# Connecting to Your PEV Now we'll connect to real hardware! Make sure your PEV is powered ON.

Cell C.2 - Connect Code

import sys, os, time sys.path.append(os.path.dirname(os.path.dirname(os.path.abspath('__file__')))) from student_api import VESCStudentAPI vesc_api = VESCStudentAPI() if vesc_api.start(): print("Starting connection...") time.sleep(6) connected = vesc_api.get_connected_controllers() vesc = vesc_api.get_controller(74) if 74 in connected and vesc.is_connected(): voltage = vesc.get_input_voltage() print(f"Connected to VESC controller!") print(f"Battery Voltage: {voltage:.1f}V") else: print("VESC not found! Is the PEV powered ON?") else: print("Failed to start VESC system")

Section D: Reading Telemetry

Cell D.1 - Read Voltage (Fill in Blank)

# Fill in the blank to read voltage voltage = vesc._________________() # Fill this in! print(f"Battery Voltage: {voltage} V") # Answer: get_input_voltage

Cell D.2 - Read RPM

# Read motor RPM rpm = vesc.get_rpm() print(f"Motor RPM: {rpm}") # RECORD RESULTS: # Motor NOT spinning: ______ RPM # Motor spinning: ______ RPM

Cell D.3 - Read Current

# Read motor current current = vesc.get_motor_current() print(f"Motor Current: {current} A") # Will be 0 when motor isn't running - normal!

Section E: Fill-in-the-Blank Challenges

Cell E.1 - FET Temperature

# CHALLENGE: Read controller (FET) temperature temp = vesc.get_____temperature() # 3 letters print(f"Controller Temperature: {____} C") # Answers: get_fet_temperature(), temp

Cell E.2 - Motor Temperature

# CHALLENGE: Read MOTOR temperature motor_temp = vesc.get_____________() print(f"Motor Temperature: {motor_temp} C") # Answer: get_motor_temperature

Cell E.3 - Multiple Readings

# BOSS CHALLENGE: Read THREE values! voltage = vesc.____________() rpm = vesc.____________() fet_temp = vesc.____________() print("=== PEV Status ===") print(f"Battery: {voltage} V") print(f"Speed: {rpm} RPM") print(f"Controller Temp: {fet_temp} C") # Answers: get_input_voltage, get_rpm, get_fet_temperature

Section F: Matching Exercise

Cell F.1 - Instructions (Markdown)

## Matching Challenge! 1. How fast is the motor spinning? 2. How much battery do I have? 3. Is the controller getting hot? 4. How much electricity is the motor using? 5. How hot is the motor itself? Functions: A. get_motor_temperature() B. get_rpm() C. get_input_voltage() D. get_motor_current() E. get_fet_temperature()

Cell F.2 - Student Answers

# Enter your answers (letters) answer_1 = "___" answer_2 = "___" answer_3 = "___" answer_4 = "___" answer_5 = "___" print("Answers recorded! Run next cell to check.")

Cell F.3 - Check Answers

correct = {1: "B", 2: "C", 3: "E", 4: "D", 5: "A"} score = 0 for i in range(1, 6): student = eval(f"answer_{i}").upper() if student == correct[i]: print(f"Question {i}: Correct!") score += 1 else: print(f"Question {i}: Wrong. Your: {student}, Correct: {correct[i]}") print(f"\nScore: {score}/5")

Section G: Wrap-Up

Cell G.1 - Clean Shutdown

# Always clean up when done! vesc_api.stop() print("VESC connection closed safely") print("Great job today! See you in Module 2!")

Cell G.2 - Summary (Markdown)

## Module 1 Complete! What you learned: - How to use Jupyter Notebooks - How to run and stop code - What an API is - How to connect to the VESC - How to read voltage, RPM, and temperature - That the data is REAL-TIME! Coming up in Module 2: - Building a live dashboard - Continuous data monitoring - Data logging